+

An Expert System for Process Modulation

by

### Wafik El-Bardissi

## Minapharm Pharmaceuticals

wafik@minapharm.com

and

# Christian R. Huyck

## Middlesex University

c.huyck@mdx.ac.uk

ABSTRACT

Manufacturing loss is usually addressed among other quality improvement measures using statistical process control and process improvement, but rarely as a problem of its own. Expert systems applied in the manufacturing field analyse quantitative parameters to optimize process functions. In the pharmaceutical and similar industries, certain criteria render this approach not applicable alone to reduce manufacturing loss.  The knowledge base of an Expert System (ES) was developed in close consultation with domain experts and revealed a novel approach that implies quantification of the defects as the means to discover the possible causative factors and derive intelligent conclusions.  The Knowledge elicitation process partly uncovered the crucial role of the complex Material-Machine-Human interaction (MMHI) in inducing production yield fluctuations. In depth analysis of the product-specific manufacturing process structure correlates to the MMHI and may explain the extent of a potentially beneficial role of an ES to reduce manufacturing loss.

A prototype of a small ES was designed and implemented on 10 consecutive batches (400,000 ampoules) of a selected pharmaceutical product. Mean post-implementation loss showed a statistically significant reduction of 60 percent compared to the pre-implementation value. Inter-batch variation was also confined to an interestingly low range. Post-implementation results were further analysed to better understand the mechanism and site of action of the ES on the process structure. The analysis cited possible refinement opportunities and provided a rationale that warrants further evaluation of an integrated pattern recognizer.

Introduction

Current approaches dealing with manufacturing loss focus on setting a minimum percentage of loss for each product and investigating only those batches that significantly go beyond such percentage. Statistical process control (SPC) methods are described [Smith, 1998] primarily as trouble indicators.  SPC would, therefore, not improve output nor minimise loss. Traditional process improvement techniques modify process steps but do not contemplate the continual batch-to-batch variations due to  Material-Machine-Human interaction  (MMHI). The absence of a pragmatic approach to deal with the latter formed the basis of this work: evaluate the efficacy of an Expert System (ES) to significantly reduce manufacturing loss in a pharmaceutical plant and hence, contribute to equally significant cost savings.

Methodology

The nature of the problem was evaluated according to the framework of testing the fit of an ES as proposed by Waterman [Waterman, 1985]. It was found to match the set criteria for requirements, appropriateness and justification.

The nature of the problem imposed the selection of the experts. The development team had to  include an expert from the production department and an expert from the engineering department. These people acted as knowledge providers, and one of the authors acted as the knowledge engineer. Both experts complied with the psychological characteristics as advocated by several authors in this field. [Goyal et al., 1985]. The synthesis rules and the description of the defects are an example of what was pertinent to the production expert, while analysis of the underlying cause(s) is the domain of the engineer.

Building the Knowledge Base

- Knowledge Elicitation:

A flow-chart of the selected product was used as the basis of the whole knowledge elicitation process. Both experts were asked to diagrammatically illustrate and explain, with a medium level of detail, the various steps of the relevant manufacturing process. (Fig. 1)

(Figure 1)

- PLPs identified:

Each step believed to contribute to any loss was identified, marked PLP (Potential Loss Point) on the chart and numbered according to the direction of material flow.  Some of the steps marked represent some type of obligatory loss such as a certain quantity of ampoules that is analysed by the Quality control as part of the in-process control. Such obligatory loss was marked OLP (Obligatory loss point) to serve for the calculation of the  theoretical output and compare it to the actual output to determine the loss.

c) Thorough analytical procedure of each PLP aimed at:

1- identifying the manifestation (defect or symptom)

2- defining the underlying cause(s)

3- setting the threshold frequency and relating the number of occurrences and their ranges to the

possible underlying causes, if such relations exist.

4-  setting the recommended actions to cure the state.

d) Creating the rules:

At this point, the rules were created. Most  rules were based on the defect or symptom, and the quantity of the defect.  The defects were classified into five types, D1-D5.  The rules identified underlying causes and recommended changes to the manufacturing process to cure the problem.

e) Defining the Input:

A list of all the data required by the system was prepared. A specific document, production loss form, was designed to disclose all data. The output parameters of each batch formed the input of the ES to provide the recommendations for the batch to follow; this was a repetitive process for 10 batches. The batch size is approximately 40,000 ampoules, so the total number of ampoules included in the trial were around 400,000 ampoules.

f) Developing the Prototype:

The rules were entered into the ES shell.  Inferencing was controlled via backward chaining.  The system was exposed to test examples and its performance was compared to the decisions of human experts. During the normal manufacturing process, each batch is analysed for number and types of defects. Properties of the materials used in production are also recorded. This information provided the input to the ES. If a batch had a significant number of defects of a certain type, the ES modified the manufacturing process to reduce that defect on the next batch.

Results

The effectiveness of the ES was tested based on the statistical analysis of the percentage loss pre- and post-implementation of the process. The pre-implementation data are acccumulated from retrospective analysis of the percentage loss of 10 batches that were found also to be in accordance with the average loss of the product over the last 2 years.

Pre-implementation manufacturing loss:

The average total percentage loss in the pre-implementation phase was 3.67% for the last 10 batches, compared to 3.8% for a number of 80 batches manufactured during last 24 months. The behaviour of the pre-implementation curve reflected considerable inter-batch variation (fig. 2a).

(Figure 2a)						(Figure 2b)

Post-implementation total manufacturing loss:

Total manufacturing loss percentage (fig. 2b) decreased significantly  (p=0.005) to 1.45% representing a 60% drop versus the pre-implementation phase. Following the implementation of the ES recommendation, the batch to batch reduction in loss became more obvious with smoothing of the curve and minimal inter-batch variation.

Figure 3

The quantity of each defect was plotted versus each batch to monitor the regression over time ( fig. 3). The baseline quantity at batch 1 was compared to the mean of batches 2-10. All defects except one showed statistically significant reduction in the number of lost ampoules.

Causal-effect relationship analysis:

A causal-effect relationship analysis was conducted on one defect where the results showed statistically significant reduction in loss and the one defect that showed no such reduction. Any observation suggestive of the absence of a causal-effect relationship was subject to a review of the relevant rules. The rationale of this approach was to observe the correlation between the ES recommendations and measured responses over time, hence, reflect the reproducibility of the former. This would provide, in addition to the objective evaluation, some subjective evidence of the system responsiveness, sensitivity and predictability.

Discussion

Our results demonstrate that implementing an AI tool, in our case an ES, is able to successfully modulate the response of a pharmaceutical manufacturing process to the ES recommendations. This response was particularly translated into significant reductions in manufacturing loss. The value of such reduction is augmented notably and correlatively with maximising productive assets utilisation (people, facility, machines) as advocated and implemented in modern manufacturing environments [Veleris et al., 1998] to provide acceptable return on assets (ROA). The manufacturing loss value in such case is worth the product ex-factory price. Regardless of the capacity utilisation level in any organisation, worldwide-accepted loss percentages are considered in the cost structure of pharmaceuticals and any reduction thereof, can be translated directly into profit gains.

In our study the implementation of the ES recommendation has lead to a statistically significant drop (60%) in the mean manufacturing loss. Furthermore, there was a significant reduction in inter-batch variation. Using statistical terms, the manufacturing process is said to be 'capable', with minimal variations embedded in the process during the post-implementation phase.

The knowledge elicitation process represents one of the pitfalls of acquiring knowledge from the human expert. The knowledge elicitation bottleneck did not seem to be as such. Highly developed perceptual abilities, the ability to simplify , the automaticity and the communication skills as recommended [Shanteau, 1996] were just a few examples among many others that characterized both experts. The success of the ideas generated was secondary to the interactive nature of the knowledge elicitation process and the in-depth analysis of the knowledge including the attempts to format it into rules.  Furthermore, some traditional process improvement measures were implemented as by-products of the knowledge elicitation process with no relevance whatsoever to the ES.

Expert systems and other intelligent techniques have been widely applied to many fields in manufacturing. Our review of the literature revealed only a few applications in the pharmaceutical or chemical industries [Alford et al., 1999].  On the other hand, many expert systems were developed for manufacturing applications such as metal fabrication, grinding, injection molding, assembly and others [Liebowitz, 1998].

There are certain criteria typical to the pharmaceutical industry and other process industries:

1- The complex synthetic nature of any product containing multiple ingredients, each portraying diverse chemical and physical characteristics.

2- The nature of the industry characterized by mass production of small delicate units (millions of pills, capsules and ampoules) realized by enormous machine speeds.

3-  Difficult defect  identification and no rework of defected items possible.

4- The irrefutable quality of a product, once released. This is realized by strict inspection and in process sampling regimens as well as quality assurance measures.

These criteria can explain the suitability of an ES approach mainly based on defect quantification for the desired purpose.  Defect quantification is based on the probabilistic nature of the dynamic element interaction and synchronization liable to bring about a given defect. Currently, real time control applications built within the machines are not feasible. Consequently, the ES is run after each batch.  The output of the system is then used to modulate the manufacturing process.

Further analysis of the manner in which individual defects behave in response to the ES recommendations reveals the following facts:

1- Causal-effect relationship was evident in the selected case showing statistically significant reduction and absent in the other not showing such reduction. It does confirm the reproducibility of the ES recommendation when effective. It also explains the mechanism of action by which the ES exerts its effects. The rules consider a threshold quantity for each defect above which the recommendations are issued. Since the recommendations are purely qualitative (reduce speed of machine, increase oxygen supply, increase distance), it can be foreseen that a decline may or may not be stepwise, depending on how the recommendations are implemented by the responsible. The ES recommends the machine speed to be decreased but does not indicate to which extent. So depending on  implicit  process phenomena, which may vary with each batch, the same action may produce either an immediate or a delayed decline to below threshold. The response is thus predictable while the time to reach its maximum is not.

Different ranges may reflect different causes that contribute to a given defect. Quantification towards the higher number of defects does not exclude a possible contribution of the causes of a smaller number of defects.

2-Most defects have started with an initially high baseline. This means that changing the machine set up from one product to the other will commonly result in a start-up peak in the first batch until the ES becomes active. Modern manufacturing trends enhance just-in-time production (JIT) to minimize inventory levels. This requires a certain degree of flexibility. Flexibility implies rapid response to customer demands and frequent modification of production plans, which in turn will result in multiple start-up peaks.

3-Different defects have shown different frequency of peak occurrences, which may or may not be consistent with the defect type.

4-The implementation of the ES did not show any statistical significance for D5 (unwelding without neck). The system development methodology advocated revising the rules in such case. This was carried out with some modification that may have caused a decline of the two following batches to below threshold, a probable secondary effect to the modification. It may also indicate, in case of the persistent absence of a causal-effect relationship that some defects will not respond to the ES recommendations irrespective of the correctness of the related knowledge representation.

The above discussion demonstrates that individual defects are prone to peaks that can be reduced but not prevented via the ES. Resistant defects may also exist. This is particularly important as it indicates that if such peaks can be prevented and resistant defects treated, a further significant reduction of the total loss can be realized.

Future Work

Our experimental setting was based on a rational approach that contemplated the granularity of a pharmaceutical manufacturing process. A distinction of two main components of its structure was identified as follows:

Coarse structure:

This can be represented by explicit knowledge deducted from the experts because the coarseness of its granularity renders the cause of any variations enhancing the manufacturing loss clearly visible to the expert.  The MMHI here is responsible for the variation but is clearly non-specific. The rules in the knowledge base are tailored to the manufacturing process. They cannot consider any variations secondary to the specific chemical or physical characteristics of the ingredients, nor the results of their interaction with the machines at different phases. The ES targets this component of the process structure.

(Figure 4)

Fine structure:

The fine granularity of the process makes the knowledge contained therein poorly visible, if at all. This state is further complicated by variations in the material characteristics from one batch to the other, still within acceptable quality ranges. At this level the MMHI depends on a specific product and plays a key important role in influencing the amount of manufacturing loss resulting therefrom. Figure 4 shows the two components of the process. It clearly demonstrates the granularity in relation to a tree-structured representation of the process, which can be carried out to as many levels as are needed by the product. The levels display the breakdown of each of the main constituents of the MMHI. For the machine, this means the number of steps and the complexity of each step. For the material it means the number of involved elements, the physical and chemical characteristics of each. Finally it is the interaction at the lowest level of the tree that translates into the production of defective units. The depths as well as breadth of the tree determine the complexity and granularity of the process and hence, the extent of potential contribution to the manufacturing loss.

The ES cannot directly influence the fine structure of the manufacturing process that embeds dynamic non-linear phenomena that are too complex and product specific to be easily described by analytical methods or empirical rules.  It does indirectly affect the fine structure by acting on the coarse structure.

However, each manufacturing run provides data on machine parameter settings, chemical properties and failure rates.  This data could be used to train a Pattern Recogniser that could select good initial parameter settings for the machines.  Consequently, the first batch of a given run would have fewer defects and would reduce or even eliminate the initial manufacturing loss peak. If a pattern recognition tool could be integrated into the process, the ES could dynamically interact with the manufacturing process to further reduce loss.

Conclusion

The preliminary results reveal a beneficial role of an expert system to minimise manufacturing loss in the pharmaceutical or any other industry of the same nature.  The nature of the industry has imposed a different approach to building the knowledge base of the ES. Traditional methods used in manufacturing that analyse quantitative parameters to derive intelligent conclusions were not applicable due to industry specific criteria..  Consequently, defect quantification was used to suggest the cause of the problem.  The ES recommended a treatment of the problem.

The Knowledge Elicitation process contributed to a better understanding of the manufacturing process.  A coarse structure represents the non-specific MMHI.  It is independent of a specific product and depends on the framework of the manufacturing process.  This is the site of a direct action of the ES.  A fine structure portrays the specific MMHI.  It depends merely on the specific property of the product components and may represent a potential site of action for a pattern recogniser.

While the role of the pattern recogniser remains to be investigated, an anticipated success will warrant further research to explore new applications for this model in different process industries. Our model will hypothetically reduce the role of material variation to a minimum. With the advent of automation and unattended manufacturing, it may  be used as an objective tool, together with other parameters, to evaluate the performance of expensive machinery use in the pharmaceutical industry or other process industries.

Bibliography

- Alford J., et al., Real rewards from artificial intelligence. The International Journal for Measurement and Control. April 01 (1999)

- Goyal S.K., et al., “COMPASS: An Expert System for Telephone Switch Maintenance.” In Expert Systems, Learned Information, Inc., Medford, NJ. (1985)

- Liebowitz J., The handbook of Applied Expert Systems. CRC Press, Boca Raton, FL. (1998)

- Shanteau J., Psychological characteristics of expert decision makers, in Proceedings Symposium on Expert Systems and Audit Judgement, Univ. of Southern California, LA, February 17-18 (1996)

- Smith G.M., Statistical Process Control and Quality Improvement, pp. 6, Prentice Hall, New Jersey (1998)

- Veleris J. and Park J., Maximizing capacity through integrated problem solving. APICS, The Performance Advantage, 8(5) pp. 58-62 (1998)

- Waterman D.A., A guide to expert systems. Reading, MA; Addison-Wesley (1985)